Statistical Analysis of Line Coding for Digital Communication

Students Information: 
1-  باسل شريف حميد مصطفى 91240202
2- محمد حمدي فتحي 91240651
3- محمد جمال جمعه السيد 91240645
4- كيرلس سمير خليل  91240586

Academic Year: 2025/2026
Course: Communication, Digital Communication Part

Supervised By: Dr.  Mohamed Nafie

# 1. Introduction

In modern communication systems, the traditional paradigm of hardwired analog signal processing is being superseded by the Software Defined Radio (SDR) approach. This shift represents a fundamental evolution in how we conceive, generate, and process signals. By bridging the gap between digital data and physical waveforms through software, engineers can dynamically modify modulation and coding schemes without the necessity of physical hardware redesigns.

The primary objective of this project is to explore these SDR principles by implementing a software-based transmitter that interfaces with the digital domain. Rather than relying on analog circuitry to define signal characteristics, we utilize software logic to generate and analyze specific line coding schemes: Unipolar, Polar NRZ, and Return to Zero (RZ).

These line codes are essential for converting raw binary data into voltage waveforms suitable for transmission. However, beyond mere generation, this project investigates the statistical properties of these waveforms. In a practical communication system, transmission is subject to randomness-both in the data content and the timing of the signal. Consequently, we treat the output of our software transmitter as a random process and evaluate it through an ensemble analysis.

The scope of this report encompasses the following core investigations:

- Waveform Synthesis: Developing robust algorithms to create an ensemble of 500 waveforms for each specified line code.

- Statistical Analysis: Computing the ensemble mean and autocorrelation to understand the underlying signal behavior.

- Property Verification: Evaluating the processes for Wide-Sense Stationarity and Ergodicity to determine if time-averaged results from a single realization are representative of the entire ensemble.

- Spectral Analysis: Estimating the bandwidth of these schemes using Fast Fourier Transform (FFT) techniques to assess spectral efficiency.

# 2. Data Generation

The foundation of the simulation is the creation of a stochastic ensemble. Rather than simulating a single instance, we utilize a Monte Carlo-style approach, generating a large matrix of independent realizations.

2.1. Stochastic Initialization: We generate 500 independent bitstreams. This is critical because a single waveform is insufficient to describe the statistics of the entire process. By utilizing an ensemble, we ensure that our calculated averages converge toward the theoretical properties of the signal.

2.2. Ensemble Matrix Structure: The ensemble is stored as a 2D matrix where each row represents a unique realization of the transmitter's output. This structure is essential for the later stages of the report, as it allows us to perform column-wise operations (to calculate the ensemble mean) and row-wise operations (to analyze autocorrelation properties).

%% Simulation Setupclear; clc; close all;

% Parameters

num_bits = 100;

num_waveforms = 500;

samples_per_bit = 7;

% Ensemble Initialization% 500 waveforms, each with 100 bits * samples_per_bit

ensemble = zeros(num_waveforms, num_bits * samples_per_bit);

for i = 1:num_waveforms

% Generate 100 random bits for each waveform

Data = randi([0 1], 1, num_bits);        %

% Store the resulting waveform in the ensemble

ensemble(i, :) = Tx_out; end

Code 1: Data Generation

2.3. Simulation Consistency: By defining the parameters (num_bits, num_waveforms, samples_per_bit) at the start, we ensure that the entire system-from time-domain generation to frequency-domain FFT analysis-remains synchronized and consistent across all statistical investigations.

# 3. Creation of unipolar ensemble

## 3.1 Methodology

% Unipolar (0 → 0, 1 → A)

Tx = Data * A;

Tx2 = repmat(Tx, samples_per_bit, 1);

Tx_out = reshape(Tx2, 1, []);

Code 2: Unipolar  Creation

In the Unipolar method, the ensemble is created by mapping bits directly to voltage levels 0 and A.
Your code utilizes Tx = Data * A to perform this mapping. The repmat function then creates the pulse width by repeating each bit value samples_per_bit times, and reshape flattens the matrix into a single waveform vector. This process is repeated 500 times to populate the rows of the ensemble matrix.

Figure 1: Frequency for Unipolar

# 4. Creation of Polar NRZ ensemble

## 4.1 Methodology:

Figure 2: Frequency of NRZ

Code 3: NRZ Creation

% Polar NRZ (0 → -A, 1 → +A)

Tx = ((2 * Data) - 1) * A;

Tx2 = repmat(Tx, samples_per_bit, 1);

Tx_out = reshape(Tx2, 1, []);

The ensemble for Polar NRZ is created using a symmetric bipolar mapping. Your code implements this via
((2 * Data) - 1) * A, which effectively transforms the binary input range {0, 1} to {-1, 1} before scaling by amplitude A. Similar to the Unipolar method, repmat and reshape are applied to expand each bit into the
required sample count, forming the continuous waveform. This ensures that the generated ensemble contains only
voltage levels {-A, +A}.

# 5. Creation of RZ ensemble

## 5.1 Methodology

% RZ (Return to Zero)

Tx = ((2 * Data) - 1) * A;

Tx_out = zeros(1, num_bits * samples_per_bit);

for k = 1:num_bits

start_idx = (k-1)*samples_per_bit + 1;    Tx_out(start_idx:start_idx+3) = Tx(k);

% first half

Tx_out(start_idx+4:start_idx+6) = 0;

% second halfend

The RZ ensemble creation differs because it requires time-domain partitioning within the bit period. Your implementation uses a for loop to iterate through each bit. The mapping logic assigns the voltage (+ A) to the first half of the samples (start_idx to start_idx+3) and explicitly forces the voltage to 0 for the second half (start_idx+4 to start_idx+6). By filling these indices directly, your code constructs a non-uniform pulse shape that returns to the baseline, which is stored row-by-row into the ensemble matrix.

Code 4: RZ Creation

Figure 3: Frequency of RZ

# 6. Applying Random Time shifts

## 6.1 Stochastic Initialization:

By generating num_bits + 1 (an extra bit), we create a buffer. This prevents the signal from being shortened when we shift it to the right.

## 6.2 Random Delay Injection:

The randi([0, max_shift]) function acts as a "jitter" generator. It effectively assigns a unique, random start-time to every waveform in the ensemble. This fulfills the project requirement that "each waveform should start from a random initial start."

## 6.3 Signal Truncation:

We use the indices (1 : num_bits * samples_per_bit) to slice the  shifted waveform back to your target matrix dimensions. This ensures that even though every waveform starts at a different relative time, they all possess the same length, which is crucial for subsequent statistical operations like calculating the ensemble mean and autocorrelation.

## 6.4 Statisical Impact

With this shift applied, your ensemble mean and autocorrelation will reflect the "asynchronous" nature of a true communication channel. This makes your stationarity and ergodicity findings more robust, as you are accounting for both data-dependent randomness and timing-dependent randomness.

# 7. Statistical Mean

## 7.1 Content:

The statistical mean represents the average value of the random process at each time instant across all realizations in the ensemble. In this project, the mean is computed manually using the definition:

μ= 1N× ∑ xit ,  M is the number of waveforms (500)

where:

- Nis the number of waveforms (500),

- xi(t)is the value of the i-th waveform at time t.

The mean is calculated column-wise across the ensemble matrix, resulting in a time-dependent mean signal. For Unipolar signaling, the expected mean is approximately equals, while for Polar NRZ it is approximately zero due to symmetry. For RZ, the mean is reduced due to the return-to-zero behavior. 
μ= A2 ,  A is the maximum voltage representing Logic '1'

Figure 4: Statistical Means

# 8. Stationarity Analysis

## 8.1 Content:

A random process is said to be Wide-Sense Stationary (WSS) if its statistical properties, such as mean and autocorrelation, do not change with time.

To verify stationarity, the variance of the statistical mean over time is computed:

Var = 1N ×∑ (m(t) - μ)2

If the variation is small relative to the mean value, the process can be considered approximately stationary.

In this project, the mean exhibits small fluctuations due to finite sample size, but remains approximately constant, indicating that the generated signals are stationary in the wide sense.

# 9. Ensemble Autocorrelation Function

## 9.1 Content:

The ensemble autocorrelation function measures the similarity between a signal and a time-shifted version of itself across all realizations.

Figure 5: Stationarity Analysis

Figure 6: Stationarity Analysis

It is defined as:

R_x(τ) = 1N ∑ x(n) x(n+τ)

The autocorrelation is computed manually for each waveform and then averaged over the ensemble.

Figure 7: Autocorrelation Functions

The resulting function is symmetric around zero lag and reaches its maximum at τ=0, indicating maximum similarity when the signal is not shifted.

For Polar NRZ, the autocorrelation is centered around zero, while Unipolar shows a positive offset due to its DC component.

# 10. Time Mean and Autocorrelation of a Single Waveform

## 10.1 Content:

The time mean is calculated for a single waveform by averaging its values over time:

μ= 1N× x(n)

Similarly, the autocorrelation of a single waveform is computed using the same shifting and multiplication process.

Figure 8: Aurocorrelation for a single waveform

This section demonstrates how a single realization behaves compared to the ensemble.

But the Time Mean for one waveform is in Figure5 within the Stationarity analysis.

# 11. Ergodicity Analysis

## 11.1 Content:

A process is said to be ergodic if the time average of a single realization equals the ensemble average.
To verify ergodicity, the time mean of one waveform is compared with the ensemble mean:

Error = |time_mean - ensemble_mean |

If the difference is sufficiently small, the process is considered ergodic.

In this project, the relative error between the two values is small, confirming that the process is approximately ergodic. This means that a single waveform can represent the statistical behavior of the entire ensemble.

Figure 9: Ergodicity Analysis

# 12. Power Spectral Density with BW Estimation

## 12.1 Content:

The Power Spectral Density (PSD) represents how the signal power is distributed over frequency.
It is obtained by applying the Fast Fourier Transform (FFT) to the signal: S(f) = |FFT(x(t))|

The PSD provides insight into the frequency components of each line coding scheme.
Unipolar signals show a strong DC component, while Polar NRZ reduces the DC component. RZ signals occupy a wider bandwidth due to faster transitions.

While the bandwidth of the transmitted signal is estimated from the frequency spectrum by identifying the frequency range where the signal power is significant; A threshold is applied to determine the effective bandwidth:

BW = f_max - f_min

Figure 10: Frequency Spectrums

RZ signaling typically has the largest bandwidth due to its rapid transitions, while NRZ has a narrower bandwidth.

Estimated Bandwidth Can be seen through Figure8 in the Ergodicity Analysis.

# 14. MATLAB Implementation

## 14.1 Content:

The entire system is implemented in MATLAB, including:

- Random bit generation

- Line coding (Unipolar, NRZ, RZ)

- Ensemble generation

- Statistical analysis (mean, autocorrelation)

- Stationarity and ergodicity checks

- Frequency analysis and bandwidth estimation

The code is structured using modular sections to improve readability and debugging.

The Source Code for this Project is in a GitHub repository for well documentation and history

# 15. Conclusion

## 15.1 Content:

This project successfully demonstrated the generation and statistical analysis of digital line-coded signals using a software-defined approach.

The results confirm that:

- The generated processes are approximately stationary

- The signals exhibit ergodic behavior

- Each line coding scheme has distinct statistical and spectral properties

This analysis highlights the importance of statistical methods in understanding communication signals and evaluating system performance.